MEMORANDUM

TO: Management
FROM: Document Specialist
DATE: October 4, 2026
SUBJECT: Total Cost of Ownership (TCO) Analysis: Diesel vs. Battery-Electric Class 8 Tractors

--------------------------------------------------

This memo provides a 7-year Total Cost of Ownership (TCO) comparison between traditional diesel Class 8 tractors and battery-electric (BEV) alternatives for our regional routes (under 250 miles/day, 60,000 miles/year per truck).

7-Year TCO Comparison (Per Truck)

| Cost Component | Diesel Tractor | Battery-Electric (BEV) |
| --- | --- | --- |
| Purchase Price | $185,000 [S2] | $420,000 [S2] |
| Federal Incentives | $0 [S2] | $0 [S2] |
| Indiana Incentives | $0 [unverified] | $0 [unverified] |
| Energy Cost (420k miles) | $228,000 | $100,800 |
| Maintenance (420k miles) | $63,000 | $37,800 |
| Charging Infrastructure | $0 | $50,000 [unverified] |
| Total 7-Year TCO | $476,000 | $608,600 |

Assumptions and Citations

Mileage: 60,000 miles/year for 7 years (Total: 420,000 miles).

Diesel Efficiency: 7.0 MPG [S1].

Diesel Fuel Price: $3.80/gallon [unverified].

BEV Energy Efficiency: 2.0 kWh/mi [S1].

Commercial Electricity Rate: $0.12/kWh [unverified].

Diesel Maintenance Cost: $0.15/mile [unverified].

BEV Maintenance Cost: $0.09/mile [unverified].

Federal Incentives: Confirmed as $0 due to cancellation of IRA 45W tax credit [S2].

Infrastructure: Estimated $50,000 per truck for charging installation [unverified].

Recommendation

Based on the current financial analysis, it is recommended to continue using diesel tractors. The Battery-Electric alternative results in a significantly higher Total Cost of Ownership, costing an additional $132,600 per truck over a 7-year period. The higher purchase price and infrastructure costs are not sufficiently offset by energy and maintenance savings, especially given the lack of available federal and state incentives in 2026.